Modbus Slave ActiveX Control

Following is a concise user’s manual for the operation of  the Modbus Slave OCX, (mbSlaveV7), control

I.	Overview

II.	Modbus_slave Control Description

A.	Properties

B.	Methods

- Events

- API Reference

III.	Visual Basic Example

Modbus Slave (V7) OCX Overview

MbSlaveV7.ocx contains a 32-bit custom control, which allows Visual Basic and other OLE Container applications to quickly, and easily interface user-defined data points to a modbus master device connected to the PC.  The modbus master may be connected directly to a PC COM port or accessed via a network connection using modbus/TCP.   The mbSlaveV7 control provides a simple API for establishing a connection and defining data points which are to be made available to any master device according to the modbus communications protocol.  The control handles all message formatting and interaction with the Windows COM and network drivers.  The only responsibility of the controlling application is to supply a pointer to the data description which is to be exposed to the connection.

Operation of the mbSlaveV7 control to simulate modbus data involves a two-step process.  First, the controlling application must define and open the physical connection to the modbus.  This involves setting various properties such as BaudRate, Parity, etc. and issuing either the OpenSerial() or OpenTCP() method.

The second step involves defining the data to be exposed to the modbus by issuing the CreateModbusData() method followed by one or more of the WriteModbus methods, (WriteModbusRegister, WriteModbusFloat, or WriteModbusCoil).  Assuming successful implementation of the above, data from within the user program is now made accessible to any connected modbus master device.  The control can support multiple connections and define multiple arrays of data to be accessed.

Distribution

The mbSlaveV7 control is distributed by WinTECH Software Design as a time-limited demo control that allows you to freely trial the operation before purchasing a developer license.  The demo control may be integrated with Visual Basic to build modbus master applications that run for 30 minutes at a time.  During this 30-minute trial, the control is fully functional and the application has full access to one or more modbus connections.  At the end of 30 minutes, the unlicensed control will stop communicating with the modbus master(s) and the application will have to be restarted to begin another 30-minute communication period.  Purchasing a license for the control will remove all time restrictions and allow the control to be distributed with the application in a run-time environment with no time limits and no additional royalty or licensing payments.

RegSvr32

RegSvr32.exe is a DOS application supplied by Microsoft for making additions and changes to the Windows Registry.  Before the mbSlaveV7 control can be used by an application it must be properly installed into the Registry.  To do this, execute RegSvr32.exe and supply the full path name to the control as an argument.  The following command line shows how to register the mbSlaveV7 control, (assuming the file is contained in the C:\MODBUS directory):

Regsvr32 c:\modbus\mbSlaveV7.ocx

Modbus Slave (V7) Control Description

The mbSlaveV7 Control consists of properties to define the operating characteristics of a physical connection to a modbus network and methods that allow the application to define the data points to be made accessible to the modbus master(s).

The control supports the following modbus message types:

| 01 | READ COIL STATUS |
| --- | --- |
| 02 | READ INPUT STATUS |
| 03 | READ HOLDING REGISTERS |
| 04 | READ INPUT REGISTERS |
| 05 | WRITE SINGLE COIL |
| 06 | WRITE SINGLE REGISTER |
| 08 | LOOPBACK COMMAND |
| 15 | WRITE MULTIPLE COILS |
| 16 | WRITE MULTIPLE REGISTERS |

The following properties and methods define the operating characteristics of the mbSlaveV7 Control:

Properties

| BaudRate | Defines the baud rate to be used for serial connections, (300,600,1200,2400,4800,9600,19200). |
| --- | --- |
| DataBits | Defines the number of data bits, (7, 8). |
| Parity | Defines parity, (0=No Parity, 1=Odd, 2=Even). |
| StopBits | Defines the number of stop bits, (0=1 stop bit, 1-1.5 stop bits, 2=2 stop bits). |
| NodeAddress | Specifies the slave node address to simulate. Changing this property has no effect until the next CreateModbusData() method is issued. The control may be used to simulate multiple slave addresses by changing this property between calls to CreateModbusData(). |
| TransmissionMode | The control can support either ASCII (0) or RTU(1) modbus transmission modes. |

Changing any of the connection properties such as BaudRate, DataBits, etc.,  will have no effect until the next ConnectSerial() method is issued.

# Methods

| ConnectSerial ( ) | Connects the mbSlaveV7 control to a COMM Port. |
| --- | --- |
| UseDanielsProtocol ( ) | Configures the ocx for the Daniel/ENRON/Omni version of modbus. |
| ConnectTCP ( ) | Connects the control to a TCP Port. |
| CloseModbusConnection ( ) | Disconnects the control from a COMM Port or TCP Port. |
| CreateModbusData ( ) | Creates a block of modbus data points to be made accessible to connected Masters. |
| WriteModbusRegister ( ) WriteModbusCoil ( ) WriteModbusFloat ( ) WriteModbusLong() | Updates a defined data block with new values. |
| ReadModbusRegister ( ) ReadModbusCoil ( ) ReadModbusFloat ( ) ReadModbusLong() | Retrieves a value from a data block, (required if the data is written from an attached modbus Master. |
| DeleteModbusData ( ) | Deletes a data block definition. |
| MessageCounter ( ) | Obtains the count of messages received from a modbus Master. |
| ClearMessageCounter ( ) | Resets the modbus Master message counter |

Events

The mbSlaveV7 Control posts the following events to the controlling application:

| DataWrittenFromMaster | The DataWrittenFromMaster Event signifies that a block of data maintained by the control has been modified via the modbus connection. The handle of the effected data block is returned as a parameter with this event. The controlling application should compare the returned value with it’s list of defined data blocks to determine which range of addresses have been written. |
| --- | --- |
| TCPConnectionFail | The TCPConnectionFail event is posted to the controlling application if the modbus/TCP service port fails during initialization. Opening the modbus/TCP service port is an asynchronous operation. The value returned from ConnectTCP reflects the handle to be used in referencing the connection, however the port has not been completely opened upon return from the ConnectTCP() method. The controlling application should monitor the TCPConnectionFail event and attempt to retry the ConnectTCP method if it occurs. |

Modbus Slave (V7) API Reference

Long ConnectSerial (short Port, short TransmissionMode)

Example:

iHandle = mbSlaveV71.ConnectSerial  (1, 1)

(Connect using COM Port 1)

The ConnectSerial method instructs the control to open the designated serial COM port and begin servicing requests from a modbus master device.  The port is opened using the current configuration properties, (BaudRate, DataBits, Parity, and StopBits).  TransmissionMode is 0 for modbus ASCII and 1 for modbus RTU.  The value returned from ConnectSerial contains the handle to the connection, and must be maintained by the controlling application as a possible parameter passed to either CloseModbusConnection() of ResetMessageCounter().  If an error occurs during opening of the serial port, the ConnectSerial method returns an INVALID_HANDLE_VALUE, (-1).

Long ConnectTCP (short Port)

Example:

iHandle = mbSlaveV71.ConnectTCP  (502)

(Connect using TCP Port 502)

The ConnectTCP() method instructs the control to open a modbus/TCP service port.  The Port parameter specifies the TCP service port number to open for modbus/TCP connections.  TCP port number 502 is the standard modbus/TCP service port specified by the protocol standard as published by Modicon.  This allows any modbus/TCP compatible master client application to connect and access the control’s data through the connected network.  The network may be physically connected to the PC or accessed via modem using the built-in dial-up capabilities of Windows.  ConnectTCP() will return a non-zero, (positive), value if the port was successfully opened.

Boolean UseDanielsProtocol (long idConnect)

Example:

IsOK = mbSlaveV71.UseDanielsProtocol  (MyConnectionHandle)

(Select the Daniel/ENRON version of modbus)

The Daniel/ENRON/Omni version of modbus uses 32-bit register values for certain addresses.  For register addresses < 6000, this version of modbus operates identical to the standard Modicon version.  Register addresses between 5000-6000 are treated as 32-bit signed integers and registers with addresses > 7000 are treated as 32-bit floating point values.  When one of these upper address registers are requested by the connected modbus master, the slave will respond with 32-bits, (4 bytes), of data.  The UseDanielsProtocol method must be esed after a connection is made but before any data blocks are created by your application.  It is not valid for TCP connections./

Long CreateModbusData (short SlaveId, short PointType, Long Address, short Length)

Example:

dHandle = mbSlaveV71.CreateModbusData (1, 3, 1, 1000)

(Create a block of data representing 1000 HOLDING REGISTERS)

The CreateModbusData() method creates an array of data within the MbSlaveV7 control that is automatically accessible to any modbus master on all active connections.  Multiple blocks of data may be created by the application to define different modbus data types and address ranges.  Each block is limited to 1000 data points.  The data within the array is uninitialized and must be written by the application using one of the Write methods, (WriteModbusRegister(), WriteModbusFloat(), or WriteModbusCoil()).

PointType refers to the modbus command used by the master to access the data and should be as follows:

## Modbus PointType

01 = COIL STATUS

02 = INPUT STATUS

03 = HOLDING REGISTER

04 = INPUT REGISTER

CreateModbusData() returns a LONG value which identifies the handle to the specified block of data.  This handle should be maintained by the controlling application to use as an input parameter to the DeleteModbusData() method or to identify data which may be written written from the master.  The DataHandle is returned as a parameter with the DataWrittenFromMaster Event.

BOOL WriteModbusRegister (short SlaveId, short PointType, Long Address, Long Value)

Example:

mbSlaveV71.WriteModbusRegister (1, 3, 1, 0)

(Write zero to HOLDING REGISTER at address 0001)

WriteModbusRegister() allows the application to update a data point contained within the MbSlaveV7 control.  Each register is updated one at a time and must be included within the address range of a previously defined DataBlock created with the CreateModbusData() method, (using PointType 3 or 4 to designate HOLDING REGISTER or INPUT REGISTER).  The Value parameter is defined as a Long to allow VB to write a 16-bit data register as a signed value in the range 0-65535.  WriteModbusRegister will return FALSE if the requested register is not contained within a previously defined DataBlock.

BOOL WriteModbusCoil (short SlaveId, short PointType, Long Address, short Value)

Example:

mbSlaveV71.WriteModbusCoil (1, 1, 1, 0)

(Write zero to COIL STATUS at address 0001)

WriteModbusCoil() allows the application to update a data point contained within the MbSlaveV7 control.  Each Coil is updated one at a time and must be included within the address range of a previously defined DataBlock created with the CreateModbusData() method, (using PointType 1 or 2 to designate COIL STATUS or INPUT STATUS).   The Value parameter is defined as a Short.  Any non-zero value will cause the addresses Coil top be set to 1.  WriteModbusCoil will return FALSE if the requested Coil is not contained within a previously defined DataBlock.

BOOL WriteModbusFloat (short SlaveId, short PointType, Long Address, Float Value)

Example:

mbSlaveV71.WriteModbusFloat (1, 3, 1, 0.000)

(Write to HOLDING REGISTERS 0001 & 0002)

WriteModbusFloat() allows the application to update data points, (two consecutively addressed modbus registers), contained within the MbSlaveV7 control.  Both addresses registers must be included within the address range of a previously defined DataBlock created with the CreateModbusData() method, (using PointType 3 or 4 to designate HOLDING REGISTER or INPUT REGISTER).  The mbSlaveV7 control updates two 16-bit modbus registers with data representing the 32-bit floating point value..  WriteModbusFloat will return FALSE if either of the requested registers is not contained within a previously defined DataBlock.  If the Daniel/ENRON/Omni version of modbus is used and the specified address is 7000 or greater, a single register address will be updated with the 32-bit floating point value.

BOOL WriteModbusLong (short SlaveId, short PointType, Long Address, Long Value)

Example:

mbSlaveV71.WriteModbusLong (1, 3, 1, 100000)

(Write to HOLDING REGISTERS 0001 & 0002)

WriteModbusLong() allows the application to update data points, (two consecutively addressed modbus registers), contained within the MbSlaveV7 control.  Both addresses registers must be included within the address range of a previously defined DataBlock created with the CreateModbusData() method, (using PointType 3 or 4 to designate HOLDING REGISTER or INPUT REGISTER).  The mbSlaveV7 control updates two 16-bit modbus registers with data representing the 32-bit signed integer value..  WriteModbusLong will return FALSE if either of the requested registers is not contained within a previously defined DataBlock. .  If the Daniel/ENRON/Omni version of modbus is used and the specified address is between 5000-6000, a single register address will be updated with the 32-bit long value.

BOOL ReadModbusRegister (short SlaveId, short PointType, Long Address, Long *pValue)

Example:

mbSlaveV71.ReadModbusRegister (1, 3, 1, MyData)

(Read the value represented by HOLDING REGISTER address 0001)

ReadModbusRegister() allows the application to read the value of a single modbus register from a DataBlock contained within the MbSlaveV7 control.  The application would need to obtain this value in the event it is written from a connected modbus master.  The pValue parameter is defined as a Long By Reference to allow VB to read a 16-bit data register as a signed value in the range 0-65535.  ReadModbusRegister will return FALSE if the requested register is not contained within a previously defined DataBlock.

BOOL ReadModbusCoil (short SlaveId, short PointType, Long Address, short *pValue)

Example:

mbSlaveV71.ReadModbusCoil(1, 1, 1, MyData)

(Read the value represented by COIL STATUS address 0001)

ReadModbusCoil() allows the application to read the value of a single modbus coil from a DataBlock contained within the MbSlaveV7 control.  The application would need to obtain this value in the event it is written from a connected modbus master.  The pValue parameter is defined as a Short By Reference.

ReadModbusCoil will return FALSE if the requested coil is not contained within a previously defined DataBlock

BOOL ReadModbusFloat (short SlaveId, short PointType, Long Address, Float *pValue)

Example:

mbSlaveV71.ReadModbusFloat (1, 3, 1, MyData)

(Read the value represented by HOLDING REGISTER addresses 0001 & 0002)

ReadModbusFloat() allows the application to read the value of two consecutively addressed modbus registers into a floating-point value.  Both registers must be included within the address range of a previously defined DataBlock created with the CreateModbusData() method, (using PointType 3 or 4 to designate HOLDING REGISTER or INPUT REGISTER).  ReadModbusFloat will return FALSE if either of the requested registers is not contained within a previously defined DataBlock .  If the Daniel/ENRON/Omni version of modbus is used and the specified address is between 5000-6000, the contents of a single register address will be provided.

BOOL ReadModbusLong (short SlaveId, short PointType, Long Address, Long *pValue)

Example:

mbSlaveV71.ReadModbusLong(1, 3, 1, MyData)

(Read the value represented by HOLDING REGISTER addresses 0001 & 0002)

ReadModbusLong() allows the application to read the value of two consecutively addressed modbus registers into a floating-point value.  Both registers must be included within the address range of a previously defined DataBlock created with the CreateModbusData() method, (using PointType 3 or 4 to designate HOLDING REGISTER or INPUT REGISTER).  ReadModbusFloat will return FALSE if either of the requested registers is not contained within a previously defined DataBlock.  If the Daniel/ENRON/Omni version of modbus is used and the specified address is between 5000-6000, the contents of a single register address will be provided

BOOL DeleteModbusData (Long DataHandle)

Example:

mbSlaveV71.DeleteModbusData (dHandle)

(Delete the block of data identified by dHandle)

The DeleteModbusData() method removes the specified block of data from the control.  The DataHandle parameter must match the value returned from a previous CreateModbusData method.  Normally, blocks of data would be created during startup, (Form Load), of an application and deleted during shutdown.  A FALSE return value from DeleteModbusData signifies that the referenced DataHandle was not contained within the control.

BOOL CloseModbusConnection (Long InterfaceHandle)

Example:

mbSlaveV71.CloseModbusConnection (iHandle)

(Close the connection identified by iHandle)

The InterfaceHandle parameter passed to CloseModbusConnection must match the LONG value returned from a ConnectSerial() or ConnectTCP() method.  This method should be used to gracefully close the modbus connection when the controlling application terminates.  The return value indicates success, (TRUE), or failure of the operation.

Long MessageCounter (Long InterfaceHandle)

Example:

Count = mbSlaveV71.MessageCounter (iHandle)

(Retrieve the message counter for the connection identified by iHandle)

The control maintains a counter of message transactions which occur over each connection.  These counters are accessible by the controlling application via the MessageCounter() method.  The control only increments the counter for a connection whenever a valid data message is received from a connected master device.

void ClearMessageCounter (Long InterfaceHandle)

Example:

mbSlaveV71.ClearMessageCounter (iHandle)

(Resete the message counter for the connection identified by iHandle)

The InterfaceHandle parameter passed to ClearMessageCounter must match the LONG value returned from a ConnectSerial() or ConnectTCP() method.  The control maintains an internal counter for each open connection detailing the number of successful message transactions which have occured on between the control and the modbus master.  The ClearMessageCounter() method simply resets the designated counter to zero.

Visual Basic Example

An example Visual Basic project is included with the mbslvocx distribution zip file.  This example, (Project1), was designed using Visual Basic 6.0 and consists of a single VBA form containing standard text controls representing two arrays of modbus data.  Defined are twenty HOLDING REGISTERS and thirty STATUS COILS which may be read/written from a modbus master.

Default settings for the example application operate on COM Port number 1 at 9600 Baud, 8 Data bits, 1 Stop Bit and no parity.  Node address 1 and RTU Transmission mode is used.  These default settings are easily customized by accessing the mbSlaveV7 property page during design time

Operation of the Project1 example begins by opening the selected COM port when the form is loaded and defing two blocks of data for access via the connection.  If the user changes any data via an edit control, the changes are automatically reflected in the data as presented to the modbus master.  Project1 uses the DataWrittenFromMaster event to redraw the edit controls if the data is changed via the modbus connection.

' Public Declarations

' Data to be made accessible via modbus

Dim MyData1(20) As Integer

Dim MyData2(30) As Integer

' Handles returned by mbSlaveV7.ocx to represent

' the connection and the two defined data blocks

Dim MyInterface As Long

Dim Data1 As Long

Dim Data2 As Long

'

' Hide the mbSlaveV7 Control when the form is activated

Private Sub Form_Activate()

MbSlaveV71.HideControl

End Sub

'

' When the form is loaded, establish the connection

' and create two modbus data blocks.

' Update the data blocks with data from MyData1 & MyData2

Private Sub Form_Load()

Dim i As Integer

Dim tf As Boolean

' To connect as serial server use following line

MyInterface = MbSlaveV71.ConnectSerial(1, 1)

' To connect as modbus/TCP server use following line

'MyInterface = MbSlaveV71.ConnectTCP(502)

' Create & Update the HOLDING REGISTER data block

' Slave Node 1, HOLDING REGISTERS addressed from 0101-0121

Data1 = MbSlaveV71.CreateModbusData(1, 3, 101, 20)

For i = 0 To 19

tf = MbSlaveV71.WriteModbusRegister(1, 3, 101 + i, MyData1(i))

Next i

'

‘Create & Update the STATUS COIL data block

' Slave Node 1, STATUS COILSS addressed from 0001-0031

Data2 = MbSlaveV71.CreateModbusData(1, 1, 1, 30)

For i = 0 To 29

tf = MbSlaveV71.WriteModbusCoil(1, 1, 1 + i, MyData2(i))

Next i

End Sub

' The user has clicked one of the displayed COIL values

' Update the MyData2 array, (local copy), as well as the

' ocx buffer

Private Sub Check1_Click(Index As Integer)

Dim tf As Boolean

MyData2(Index) = Check1(Index).Value

tf = MbSlaveV71.WriteModbusCoil(1, 1, 1 + Index, MyData2(Index))

End Sub

'

' The user has changed the value of one of the displayed REGISTER values

' Update the MyData1 array, (local copy), as well as the

' ocx buffer

Private Sub Text1_Change(Index As Integer)

Dim tf As Boolean

MyData1(Index) = Text1(Index).Text

tf = MbSlaveV71.WriteModbusRegister(1, 3, 101 + Index, MyData1(Index))

End Sub

'

' One of the defined data blocks has been written from a connected

' modbus master.  Determine which data block is involved and update the

' local data array from the internal array contained within the ocx

Private Sub MbSlaveV71_DataWrittenFromMaster(ByVal hModbus As Long)

If (hModbus = Data1) Then

Call Refresh_Regs

End If

If (hModbus = Data2) Then

Call Refresh_Coils

End If

End Sub

' Procedure to read the modbus data buffer, (HOLDING REGISTERS)

' from the mbSlaveV7.ocx buffer and update the VB Form's display

' This procedure is called in response to a DataWrittenFromMaster

' event from the control

Public Sub Refresh_Regs()

Dim i As Integer

Dim temp As Long

Dim tf As Boolean

For i = 0 To 19

tf = MbSlaveV71.ReadModbusRegister(1, 3, 101 + i, temp)

MyData1(i) = temp

Text1(i).Text = MyData1(i)

Next i

End Sub

' Procedure to read the modbus data buffer, (STATUS COILS)

' from the mbSlaveV7.ocx buffer and update the VB Form's display

' This procedure is called in response to a DataWrittenFromMaster

' event from the control

Public Sub Refresh_Coils()

Dim i As Integer

Dim tf As Boolean

For i = 0 To 29

tf = MbSlaveV71.ReadModbusCoil(1, 1, 1 + i, MyData2(i))

Check1(i).Value = MyData2(i)

Next i

End Sub

'

' Simple Timer routine to obtain and display the message counter

' from the mbSlaveV7 ocx

Private Sub Timer1_Timer()

Text2.Text = MbSlaveV71.MessageCounter(MyInterface)

End Sub

'

' Cleanup the data and close the connection when the form unloads

Private Sub Form_Unload(Cancel As Integer)

Dim tf As Boolean

tf = MbSlaveV71.DeleteModbusData(Data1)

tf = MbSlaveV71.DeleteModbusData(Data2)

tf = MbSlaveV71.CloseModbusConnection(MyInterface)

End Sub