WinTECH Software Design

OPCXchg

Data Exchange Utility for Industrial Applications

I.	Introduction

A.	Purpose of this manual

B.	Software Distribution Method

C.	Basic Software License

D.	How to contact WinTECH Software Design

II.	Overview of OPCXchg

- Functional Description

- Compatible Devices

C.	Installation

D.	Running the Demos

III.	Applications for the software

A.	Modbus OPC Server Application

B.	OPC Client to Modbus Slave

C.	OPC Server to OPC Server

D.	Modbus Master to Modbus Master

E.	Modbus/OPC to Visual Basic/Excel

F.	Custom OPC Server/Client Designs

G.	Device Configurator

IV.	Operation

A.	Establishing Device Connections

B.	Building the Tag List

C.	Logic Blocks

D.	Graphical Display Objects

E.	Working with Forms

- Saving/Restoring Configuration

- Other WinTECH Products

- ModScan32

- ModSim32

- Modbus ActiveX Controls

- OPC Developer Tools

Purpose of this manual

This manual represents a composite technical description of the OPCXchg application offered by WinTECH Software Design to support data acquisition and manipulation using the modbus communications protocol and OLE for Process Control.  Topics covered include the distribution and licensing methods utilized to market the software as well as a detailed user’s manual for the application.

Software Distribution Method

The WinTECH Software Design suite of Industrial Automation products is distributed primarily via the internet.  Fully-functional demo applications are available from the following Web-Site:

http://www.win-tech.com

Each software application may be downloaded for evaluation and freely distributed among potential users without cost or obligation.  Each application is in some fashion time-limited, allowing free and unrestricted use for a pre-defined period of time.  This is usually about 3 minutes after establishing communication with a connected modbus device.  After the demo-time elapses, the software will cease to function and the application must be restarted to continue

If the software proves useful, and a user wishes to remove the time-limit from its operation, he must purchase an access code from WinTECH Software Design.  For WinTECH Applications, this access code must be entered into the initial sign-on dialog box to register the software, (one-time only).  Evaluation versions of the software are identical to commercial, (registered), versions with the exception of the access code which removes all time-limits and other registration incentives which may be included/excluded from the evaluation copy.  Once registered, the software may no longer be distributed and must remain on a single machine, (PC).  The user must conform to the software license included with the registration access code and protect the confidentiality of the application.

ActiveX controls are protected somewhat differently.  The evaluation versions of the modbus OCX controls allow full operation in Visual Basic Design Mode for up to 30 minutes.  During this time, the user may exercise a control without restriction.  Upon purchase of the control, the user will receive two licensing files from WinTECH Software Design which removes the 30-minute restriction.  ActiveX controls are licensed to be installed on one machine only to be used in a development environment.  There are no additional payments or royalty fees required to include the control in a user design to be distributed,

(run-time operation), in object form.

WinTECH Software Design License Agreement

YOU SHOULD CAREFULLY READ THE FOLLOWING TERMS AND CONDITIONS BEFORE USING THE ACCESS CODES SUPPLIED HEREIN.  USING THE SUPPLIED ACCESS CODES TO REGISTER A WINTECH SOFTWARE APPLICATION INDICATES YOUR ACCEPTANCE OF THESE TERMS AND CONDITIONS.  IF YOU DO NOT AGREE WITH THEM, YOU SHOULD PROMPTLY RETURN THE ACCESS CODES TO WINTECH SOFTWARE DESIGN WITHIN FIFTEEN DAYS OF ACQUISITION AND THE REGISTRATION LICENSE FEE PAID WILL BE REFUNDED.

WinTECH Software Design provides computer programs contained on diskettes and/or electronic distribution services, (the “Applications”) and associated help files and documentation, (the “Documentation”).  Each Application requires the use of an encryption string, (the “Access Code”), to remove certain operational restrictions contained within the Application.  WinTECH Software Design licenses the use of the Applications, Documentation and Access Codes according to the terms and conditions set forth herein.  You assume responsibility for the selection of the Applications to achieve your intended results, and for the installation, use and result obtained from the Applications.

LICENSE

1.  You are granted a personal, non-transferable and non-exclusive license to use one copy of the software contained with the Application on a single personal computer and to use the documentation and Access Code under the terms stated in this Agreement.  Title and ownership of the Applications and Documentation remain with WinTECH Software Design

2.  You, your employees and/or agents are required to protect the confidentiality of the Applications and Documentation.  You may not distribute or otherwise make the Access Codes available to any third party.

3.  You may not assign, sublicense or transfer this license and may not decompile, reverse engineer, modify, or copy the Application or Documentation for any purpose, except you may copy the Application and Access Code into machine readable or printed form for backup purposes in support of your use of the Application on a single machine.

4.  The Applications, Access Codes, and Documentation are copyrighted by WinTECH Software Design.  You agree to respect and not to remove or conceal from view any copyright, trademark, or confidentiality notices appearing on the Application or Documentation, and to reproduce any such copyright, trademark or confidentiality notices on all copies of the Application and Documentation or any portion thereof made by you as permitted hereunder.

YOU MAY NOT USE, COPY, MODIFY, OR TRANSFER THE APPLICATIONS, DOCUMENTATION, OR ACCESS CODES IN WHOLE OR IN PART, EXCEPT AS EXPRESSLY PROVIDED FOR IN THIS LICENSE.

IF YOU TRANSFER POSSESSION OF ANY COPY OF THE ACCESS CODES TO ANOTHER PARTY, YOUR LICENSE IS AUTOMATICALLY TERMINATED.

TERM

This license is effective until terminated.  You may terminate it at any time by destroying the Applications, Documentation and Access Codes along with all copies in any form.  It will also terminate upon conditions set forth elsewhere in this Agreement if you fail to comply with any term or condition of this Agreement.  You agree upon such termination to destroy the Applications, Documentation, and Access Codes together with all copies in any form.

LIMITED WARRANTY

During the first 90 days after delivery of the Access Codes to you, as evidenced by a copy of your receipt, invoice or other proof of purchase, (the “Warranty Period”), WinTECH Software Design warrants that the Application will perform substantially in accordance with the Documentation and that the diskettes on which the Applications are furnished, (if supplied), are free from defects in materials and workmanship under normal use.  EXCEPT AS PROVIDED N THIS SECTION, THE APPLICATIONS AND DOCUMENTATION ARE PROVIDED WITHOUT WARRANTY OF ANY KIND, EITHER EXPRESSED OR IMPLIED, INCLUDING, BUT NOT LIMITED TO, THE IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE.

SOME STATES DO NOT ALLOW THE EXCLUSION OF IMPLIED WARRANTIES, SO THE ABOVE EXCLUSION MAY NOT APPLY TO YOU.  THIS WARRANTY GIVES YOU SPECIFIC LEGAL RIGHTS, AND YOU MAY HAVE OTHER RIGHTS WHICH VARY FROM STATE TO STATE.

LIMITATION OF REMEDIES

1. WinTECH Software Design shall use commercially reasonable efforts to correct any failure of the Applications of which it is given written notice by you during the Warranty Period to perform substantially in accordance with the Documentation, provided such a failure can be recreated by WinTECH Software Design in an unmodified version of the Application or, if WinTECH Software Design is unable to correct such failure you may terminate the Agreement by returning the Application, Documentation, and Access Code and the License Fee paid will be refunded.

2.  WinTECH Software Design shall replace any diskette not meeting WinTECH Software Design’s “Limited Warranty” and which is returned to WinTECH Software Design with a copy of your receipt, invoice, or other proof of purchase or, if WinTECH Software Design is unable to deliver a replacement diskette which is free from defects in materials or workmanship, you may terminate this Agreement by returning the Application, Documentation and Access Code and the License Fee paid will be refunded.

IN NO EVENT WILL WINTECH SOFTWARE DESIGN BE LIABLE TO YOU FOR ANY DAMAGES, INCLUDING ANY LOST PROFITS, LOST SAVINGS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES ARISING OUT OF THE USE OR INABILITY TO USE THE APPLICATIONS EVEN IF WINTECH SOFTWARE DESIGN HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES, OR FOR ANY CLAIM BY ANY OTHER PARTY.

SOME STATES DO NOT ALLOW THE LIMITATION OR EXCLUSION OF LIABILITY FOR INCIDENTAL OR CONSEQUENTIAL DAMAGES SO THE ABOVE EXCLUSION MAY NOT APPLY TO YOU.

GENERAL

This Agreement will be goverened by the internal laws of the State of West Virginia.

YOU UNDERSTAND THAT YOU HAVE READ THIS AGREEMENT, UNDERSTAND IT AND AGREE TO BE BOUND BY ITS TERMS AND CONDITIONS.  YOU FURTHER AGREE THAT IT IS THE COMPLETE AND EXCLUSIVE STATEMENT OF THE AGREEMENT BETWEEN YOU AND WINTECH SOFTWARE DESIGN WHICH SUPERSEDES ANY PROPOSAL, PRIOR OR CONTEMPORANEOUS AGREEMENT, ORAL OR WRITTEN, AND ANY OTHER COMMUNICATIONS BETWEEN US RELATING TO THE SUBJECT MATTER OF THIS AGREEMENT.

How to contact WinTECH Software Design

The most expedient method for contacting WinTECH Software Design is via e-mail using the following addresses:

Sales support:

sales@win-tech.com

Technical support:

support@win-tech.com

WinTECH Software Design is located in the Eastern Time Zone of the United States and may be reached via phone or fax at the following number:

(304) 645-5966

The postal address is:

WinTECH Software Design

P.O. Box 907

Lewisburg, WV  24901

USA

OPCXchg Description

OPCXchg is a Windows application that allows you to access data from various third-party devices and utilize this data in a variety of ways to create graphical displays and/or exchange this data with other devices.  OPCXchg supports both modbus master and modbus slave devices as well as local and remote OPC connections.  Modbus devices may be connected serially via one or more of the PC’s COMM ports or via a TCP/IP network interface.  Data from any connected device, (or OPC application), may be exchanged with any other connected device.  A simplified user interface allows data points to be displayed within OPCXchg using a variety of built-in graphical objects.  OPCXchg also supports OLE Automation that allows data from connected devices to be accessed from Visual Basic and/or Excel.

Basic operation of the OPCXchg application consists of first establishing a connection to one or more devices, (applications), and then building an internal tag list to access various data points within each device.  Logic blocks may be defined to perform calculations on the data and/or push data from one device to another.  OPCXchg operates as an OPC Server, making the defined tag list automatically available to any OPC compliant client application.  The tag list is also made available to Visual Basic using simple OLE Automation read and write functions.

Compatible Devices

OPCXchg supports up to nine serial connections.  Each serial channel may be connected to either a single modbus master device or one or more modbus slave devices.  Baud rates are supported up to 19.2K and both the RTU and ASCII transmission modes are supported.  OPCXchg supports modbus message types 01-06, 15&16.  Data contained in modbus registers may be accessed using a variety of formats including integer, unsigned integer, scaled, and floating point.

OPCXchg also supports the modbus/TCP protocol.  Multiple modbus/TCP devices may be connected to OPCXchg using a standard network interface.

OPCXchg may also be connected to any OPC 1.1 or 2.0 compliant server.  The server may reside on the local machine or on a remote machine connected via a network interface.  Data from the connected server may be accessed and exchanged with other data within OPCXchg.  For the purposes of this manual, a connected OPC Server represents just another device.

Since the OPCXchg application is itself an OPC Server, compatible OPC Client applications may access the defined tag list to read and write data points within a connected device.

OPCXchg Installation

OPCXchg requires the installation of several support dll’s provided by the OPC Foundation.  These OPC proxy dll’s are freely available from the OPC Foundation web site at www.opcfoundation.org and include the following:

OPCPROXY.DLL

OPCCOMN.DLL

OPC_AEPS.DLL

OPCHDA_PS.DLL

These four dll’s should be installed in the \Windows\System32 directory and registered using RegSvr32.  Please consult the OPC Foundation website to obtain the latest version of these proxy dll’s and follow the installation instructions contained therein.

In addition to the above dll’s, the OPC Foundation also supplies a service lookup component required by OPCXchg to lookup the list of installed OPC Servers on the local or remote machine.  OPCENUM.exe should also be located in the \Windows\System32 directory and registered from the command line using:

C:>\Windows\System32: OPCENUM –RegServer

The OPCXchg.exe and supporting dll;s supplied by WinTECH Software Design do not require registration with Windows and may simply be extracted from the distribution zip file into a common Windows directory and executed from there.

Certain example forms supplied with the OPCXchg application require connections to sample OPC Server applications.  These servers, (OPCSvrTst1 & OPCSvrTst2), are supplied with the OPCXchg distribution set and must be registered with the Windows Registry before OPCXchg can establish a connection.  Before running the OPCXchg demos, please register the two sample servers by running each executable.  This will install the servers and allow OPCXchg, and other OPC Clients), to connect and access the sample data.

OPCXchg Demos

OPCXchg is supplied with several custom forms designed to demonstrate various operational aspects of the software.  Since the whole purpose of the OPCXchg application is to exchange data between third-party devices, these demos wouldn’t make a lot of sense without third-party devices connected.

When OPCXchg first starts up, it looks for a configuration file named “opcxchg.tst”.  If found, this file is automatically loaded.  The distribution zip file containing OPCXchg contains a sample opcxchg.tst configuration containing five different forms.  The first of which provides an overview of the application and contains “Hot Text” buttons to launch the other four displays.

The first demo screen attempts to show how data from a modbus slave may be accessed from OPCXchg and made available to third-party OPC Client applications.  In this example, OPCXchg operates as a modbus OPC Server.  Various graphical capabilities of OPCXchg are used in the demo including bargraphs and trend charts.

In order for this demo to be effective, OPCXchg must be connected to a modbus slave device via COMM port 1.  The slave should be configured as node address 01 and contain holding registers 40101-40110.  For this example, data contained within the modbus register is expected to be a scaled value between

0-9999.  The serial connection must be configured as 9600 baud, 8 Databits, 1 StopBit and No Parity.

The OPCXchg taglist for this demo screen is defined as follows:

MODBUS.COMM1.40101

MODBUS.COMM1.40102

MODBUS.COMM1.40103

MODBUS.COMM1.40104

MODBUS.COMM1.40105

MODBUS.COMM1.40106

MODBUS.COMM1.40107

MODBUS.COMM1.40108

MODBUS.COMM1.40109

MODBUS.COMM1.40110

Any third-party OPC Client may attach to OPCXchg, (WinTECH.OPC_Exchange_Server), and access data from the modbus slave using the defined tag names.  The client has both read and write access.

The second demo form attempts to show how OPCXchg can operate as a modbus slave.  In this case, data is obtained from a connected OPC Server and made available as modbus slave data to any modbus/TCP client connected to the machine.  In this example, two tags are read from the server, added together and written to a modbus slave register.  These registers may be read by any connected modbus master application.  For this particular demo, only the modbus/TCP port is opened.  To access the data from a serial port, you must first connect to one of the OPCXchg COMM ports.

This demo expects to connect to an OPC Server on the local machine named “OPCSvrTst1”.  This is a simple OPC Server provided with the OPCXchg distribution set.  OPCSvrTst1 must first be registered in the Windows Registry before OPCXchg can connect to it.  You can register OPCSvrTst1 by simply running the executable, (opcsvrtst1.exe).  The server is self registering and after being ran the first time, will be made available to any opc client including OPCXchg.

OPCSvrTst1 provides the following items, each of which is read by OPCXchg:

OPCXchg creates an internal taglist based on the items contained in OPCsvrTst1:

SVR1.CarbonMonoxide(1)

SVR1.CarbonMonoxide(2)

SVR1.CarbonDioxide(1)

SVR1.CarbonDioxide(2)

SVR1.Oxygen(1)

SVR1.Oxygen(2)

SVR1.H2S(1)

SVR1.H2S(2)

SVR1.Temp

SVR1.Rate

SVR1.Input

In addition tags are created as follows:

Total_CO

Total C02

Total O2

Total H2S

Logic blocks are linked to the first eight tags.  When the value of either “SVR1.CarbonMonoxide(1)” or “SVR1.CarbonMonoxide(2)” changes, the tag “Total CO” is calculated.  Likewise “Total CO2”,

“Total O2”, and “Total H2S” are calculated from the corresponding server items.  These total values are defined as modbus slave register tags, so whenever their values change, the change will automatically be reflected in the value read by a modbus/TCP master.  For example, the tag “Total CO” is defined as follows:

The third demo screen shows how data may be exchanged between two different OPC Server applications.

OPCSvrTst1 is used as the first OPC Server: OPCSvrTst2 is used as the second.  As before, OPCSvrTst2 must be registered with the Windows Registry before OPCXchg can connect to it.  Run OPCSvrTst2.exe once before attempting to run this demo.

OPCSvrTst2 supplies twelve items.  These are included in the OPCXchg tag list as:

SVR2.Tag1

SVR2.Tag2

.

.

.

SVR2.Tag12

Logic Blocks are connected to each SVR2 tag to update a corresponding tag in SVR1 upon data chnage.  Likewise, a Logic Block is tied to each SVR1 tag to update SVR2 if its value changes.

Using the two different server applications, you can change the value of a tag in one server and the corresponding tag in the other server should track the value.  You can also double-click on a tag value displayed within OPCXchg to update the tag in both servers.  As with all tags defined within OPCXchg, any connected OPC Client may also access and change a tag’s value.  In all cases, whenever the tag changes within one server, the corresponding tag should also change.

The last demo form supplied with OPCXchg simply lists the complete tag list and is designed to show how the data can be accessed from Excel using OLE Automation.

A sample Excel worksheet, (Book1.xls), is supplied with OPCXchg that provides the capability to read and write each tag defined within OPCXchg.  This worksheet contains three buttons and lists each tag by name.  The first button is used to connect the worksheet to OPCXchg.  The Read button simply calls an OLE Automation function to read each tag by name and the Write button is used to write the data from the worksheet to OPCXchg.

Using OPCXchg as a modbus/OPC Server

Modbus slave devices may be connected to OPCXchg via any of nine COMM ports or via a TCP connection.

As modbus data points are added to the OPCXchg tag list, these data points will be added to the scan list for the associated connection.

Each modbus tag is defined to represent a value obtained from the connected slave and assigned an itemname. As the tag is defined, the user identifies the comm port, slave node address, point type, and point address.

Modbus Register data may be accessed using different formats, including integer, unsigned, floating point and scaled.  Modbus HOLDING REGISTERS and STATUS COILS are treated as Read/Write tags within the OPCXchg OPC Server while INPUT REGISTER and INPUT STATUS values are treated as Read-Only tags.

After the tag list is defined to represent the modbus data points of interest, any OPC compliant client application may access the data.  The OPC Server Name uised by OPCXchg is “WinTECH.OPC Exchange Server”.  The server supports the “Browse Items” interface that allows a client to obtain the list of items supported.  The server also supports the hierarchical namespace and if segmented names are used to identify the tags, (i.e. Slave1.HoldingRegister.0001, Slave1.HoldingRegister.0002, etc.), the browse interface will return a tree structure as expected.

Using OPCXchg as a modbus slave

OPCXchg may be used to exchange data between third-party OPC Servers and modbus Masters.  To do this, you must define two sets of tags within OPCXchg and use Logic Blocks to move data between the two devices.

First, connect to the OPC Server and browse the published item list to create tags within OPCXchg representing the data of interest.

When an OPC Server is connected to OPCXchg, the server name will show up in the Add Tag dialog box.

You can then browse through the list of items supported by the server and select those of interest.

After building the tag list representing data obtained from the connected server, you must define corresponding tags to represent data to be exposed as modbus slave points.

For each tag defined, you must link to a Logic Block that will update the corresponding tag in the other device whenever its value changes.  In this way, any time the data point contained in the OPC Server is updated, the corresponding modbus slave tag will be updated.  If a connected modbus master writes to a defined slave tag, the value will be automatically transferred to the connected server, (assuming that the server supports write access for the tag.)

Modbus slave data tags defined within OPCXchg are accessible to any connected modbus master device.  The master may be connected to a serial COMM port or via a modbus/TCP connection.

Using OPCXchg to exchange data between OPC Servers

OPCXchg supports multiple asynchronous connections to different devices.  To exchange data between two third-party OPC Servers, you must establish a connection to each and define OPCXchg tags representing data from each server.  Again, Logic Blocks must be linked to each tag that will update a corresponding tag in the other server whenever the first tag’s value is changed.

Using OPCXchg to exchange data between modbus masters

Likewise, data may be exchanged between two connected modbus masters.  Simply connect to each device and define OPCXchg tags to represent the data.  Link each tag to a Logic Block that will be initiated on Data Change to update the corresponding tag originating from the second device.

Using OPCXchg as an Interface to Excel

OPCXchg supports OLE Automation functions to provide both read and write access to the defined tag list.  After establishing connections to various modbus and/or OPC devices and creating the list of tags representing the data of interest, Excel has complete access.  All you have to do is tell Excel where the data is located and exercise the ReadTag and WriteTag functions.

To establish the automation link between Excel and OPCXchg, simply execute the following two lines of code using VBA:

Public m_svr As Object

Set m_svr = CreateObject("OPCXchg.Document")

This creates the data link and allows Excel to use the functions exported from OPCXchg .  To read a tag from OPCXchg, use the ReadTag function:

Dim Value As Variant

Value = m_svr.ReadTag("Exchange.Tag1”)

The string argument passed to ReadTag is the tagname identifying the OPCXchg item to read.  If you want to write an item from Excel, use the WriteTag function:

status = m_svr.WriteTag("Exchange.Tag1", Value)

The first argument to WriteTag is a string representing the OPCXchg tagname.  The second argument is a Variant containing the data to write.

Using OPCXchg to create a custom OPC Server/Client

The ReadTag and WriteTag Automation functions exported from OPCXchg may also be used with Visual Basic to create fully compliant OPC Server applications.  You can create a list of tag names within OPCXchg that are defined as local user variables, (those not associated with any external device).

Using the OPCXchg Automation functions from a Visual Basic application allows you to control the values of these tags as required.  Logic within your VB App can obtain the data from custom devices and update the tags according to your particular requirements.  Each tag defined within OPCXchg is automatically made available to OPC Clients, so you have just created a custom OPC Server representing your specific data.

Likewise, custom OPC Client applications may be developed in Visual Basic to utilize data from one or more OPC Servers connected via OPCXchg.

Using OPCXchg as a Device Configurator

One of the unique aspects of the OPCXchg application is the ability to display data using a variety of built-in graphical objects.  These include the display of item values using different fonts and colors as well as bargraph items and trend charts.

Often, the user is faced with situations where two different OPC Servers from different vendors are available, each with different item names representing a common function.  For example, Vendor A may supply data using tag names such as “unit.temperature”, “unit.pressure”, and “unit flow” to represent three process variables while Vendor B may use tag names such as “temp”,

“pressure”, and “flow” to represent the same data types.  Designing an OPC Client to utilize the data would require knowledge of which vendor’s product is actually connected to the source of the data.  This is a simplistic example, but could become tedious to implement a configuration device if a large number of data tags are used.  OPCXchg allows you to design a client application, (using Visual Basic or any OLE Automation compatible system), using a common set of tag names.  After the majority of effort is expended within the client code, you can create a simple OPCXchg tag list for each Vendor’s device to cross reference specific tag names to those expected by the configuration tool.  In this way, you have created one tool that works with several different servers.

OPCXchg Device Connections

(Serial modbus)

Connections may be established between OPCXchg and modbus devices connected to any of nine available COMM ports.  Each COMM port connection may be configured as either a modbus master or slave.

When configured as a modbus master, data points will be polled from connected slave devices as defined in the OPCXchg tag list.  If the port is configured as a slave, a connected modbus master may access any slave data defined by OPCXchg.  Slave data points defined within the OPCXchg application are not tied to a specific COMM port but are accessible to any modbus master connected to the machine, including those connected via TCP.

OPCXchg Device Connections

(modbus/TCP)

OPCXchg can operate as a modbus/TCP server when connected as a Modbus Slave on a TCP Port. The user specifies which port is to be used as the modbus/TCP Service port:

When a connection is made from OPCXchg to an external modbus/TCP server, the user must specify both the machine name, (IP Address), of the service machine as well as the service port it uses for modbus communications.  The machine name, (or IP Address), is used within the tag list to identify the connection containing the data point.

OPCXchg Device Connections

(OPC Servers)

OPCXchg can connect to multiple OPC Servers located on the local machine or on a remote machine.

When an OPC connection is attempted, the software will provide a list of installed OPC servers on the specified machine.  The user selects from this list to establish the connection.  (OPC Connections must be established prior to adding OPC tags to the tag list.  OPCXchg uses the Browse Interface for a connected server to allow the user to add item references.)

OPCXchg Tag List

The tag list is the heart of the OPCXchg application.  Defined tags may represent data points from connected modbus devices, connected OPC Servers, and/or user variable points used for display and/or intermediate calculations.

The OPCXchg tag list is presented as a multi-column table.  Highlighting the name of a displayed tag will allow you to delete or modify the tag definition.  New tags may be added as desired.

The user specifies the name that identifies the tag and the tag type, (source of data).  Notice in this example that two OPC connections have previously been made, (OPCSvrTst1 & OPCSvrTst2).  The “Details” button will bring up a new dialog containing extended information to help define the tag.  This dialog will be different depending upon the specified tag type.  For example, the following details dialog is presented for the user variable data type:

:

This specifies the initial value for the tag.  User Variable tags may be changed at any time by Logic Block execution, user changes from OLE Automation connections, and/or OPC Clients connected to OPCXChg.

If the specified tag type refers to a modbus data point, the details dialog contains information relative to the identification of the point within the connected device, (slave node, point address, data type, etc.)  If the tag type refers to an OPC Server connection, the details dialog will list the available items exported from the server:

OPCXchg Logic Blocks

Logic Blocks are used within OPCXchg to initiate an action based upon the occurrence of some event.  Generally, logic blocks are tied to the value of one or more defined tags, such that they are executed whenever the value of the tag changes.  This allows one tag to update another and is the basis for exchanging data between independent devices.

The user may define an unlimited number of Logic Blocks to perform various functions and each Logic Block consists of multiple entries which are executed in sequence.  There are currently three functions that may be included in a Logic Block.

The first is an assignment operation whereby one tag may be assigned to another or two tags added together and assigned to a third.

The drop-down edit controls list each tag currently defined in the tag list.  Notice the operator list consists of the four basic math operations and a null entry.  If no operator is selected, the dialog control representing “Tag3” is not used.

The second type of Logic Block entry consists of a logical “IF’ followed by a “GOTO”.  Execution of this entry would perform a comparison operation and go to another Logic Block based on the results:

The tag defined by the “Tag1” edit control is compared to the value of the tag defined by the “Tag2” edit control and if the results match the defined operator, execution will go to the defined Logic Block and optional Label.  The operator specified may be “equal to”, “greater than”, “less than”, “greater than or equal to”, or “less than or equal to”.  If the comparison fails, execution goes to the next sequential statement in the current Logic Block.

The third entry in a Logic Block allows you to show or hide a specific OPCXchg Form.

OPCXchg Graphical Display Objects

The OPCXchg display is completely customizable.  You may create forms within OPCXchg to represent data specific to your particular application using a variety of built in graphical objects such as lines, circles, rectangles, text displays, and values obtained from the OPCXchg tag list.   Tag values may be displayed numerically or represented by bar and/or trend charts.

When the OPCXchg application is operating in “Design Mode”, (unlocked), you may place a new object on the display by holding the left mouse button down and dragging a rectangle in the approximate place you want the new object to appear.  When the mouse button is released, a pop-up dialog box will appear listing the various items that may be inserted.

Selecting an item from the list will initiate a detailed dialog in which you can define the parameters unique to the object.  For example, the dialog box defining the properties for a Static Text object looks like the following:

Properties for a Static Text object define the text string as well as the font and colors to use for drawing the text.  You may specify both the foreground and background colors, or select a transparent background.

Simple drawing items such as Rectangles, Bitmaps, and Ellipses may also be inserted on the form.  Properties for these items define the height and width of each display, line widths, colors, and an option to draw the item to the form before other objects.  This provides the capability for other types of objects to lay on top of a simple shape.

If a Data Value is selected for display, the details dialog identifies the tag name and defines parameters associated with how the value is displayed:

Again, the value may be displayed in various colors and fonts.  The “Write Enabled” option allows the user to write to a tag by double-clicking on the displayed value.

The properties defined for a bargraph appear as follows:

The tag name serving as the source of data for the bargraph is defined as well as the height and width parameters.  Bargraphs may be drawn horizontally or vertically using different colors.  The display of the bargraph is determined by scaling the defined tag between the specified high and low limits.  Options exist to display a border around the bargraph, show the high and low limits, (scaling), on the display, and allow the user to write to the tag by double-clicking on the display.

Hot Text objects are similar to Simple Text Displays except they can be linked to Logic Blocks to initiate an action when the user clicks within the Hot Text area.

Trend Chart objects may be added to the OPCXchg form and may contain up to five pens representing five different tag values:

The TimeSpan parameter specifies, (indirectly), how often data points are added to the cart.  The chart may represent time spans in minutes or seconds and the less the specified span, the quicker the data is updated.  Each data value, (pen), contained within the cart may be defined in different colors and using different scaling factors.

OPCXchg Forms

OPCXchg operates as a standard Microsoft MDI, (Multiple Document Interface), application.  New documents, (Forms), may be created, saved, and opened using the standard menu commands, (“File New”,

“File Save”, and “File Open”).  OPCXchg allows you to define certain characteristics of each form that determine how the form is displayed:

The Refresh Rate specifies the frequency at which objects contained within the form are updated.  The specified Document Name is how the form is referenced by other items contained with OPCXchg.  For example, a Logic Block entry may try to display or hide a form by referencing the name “OPCXchg_DEMO” as defined above.

OPCXchg Save/Restore

OPCXchg documents may be saved and restored individually, however, generally you will want to create a custom application which restores all data on start-up.  The Menu commands “Save All” and “Restore All” do just that.  These commands operate on all documents defined within the context of OPCXchg.  Not only are all the forms saved, but also each connection is saved along with the complete tag list and logic block tables.  After establishing all connections, defining the tag list, and creating the various display forms, use the “Save All” command to save the complete configuration.  If you specify “opcxchg.tst” as the filename, the configuration will be automatically restored whenever OPCXchg starts up.

Other WinTECH Applications

WinTECH Software Design supplies several utility applications for testing and simulating industrial communications devices.  ModScan32 is a Windows application that operates as a modbus master.  ModScan32 supports serial connections as well as modbus/TCP connections and may be used to test and verify modbus slave applications including slave data exported from OPCXchg.

ModSim32 is a modbus slave simulator and allows you to define various modbus data points for testing modbus master applications.  ModSim32 cold be used to simulate inputs into OPCXchg for testing or execution of the demo forms.

WinTECH Software Design can also provide developer tools for designing custom modbus an/or OPC applications.  ActiveX controls are available that support modbus master and modbus slave communications and complete source code toolkits are available for either OPC Server or Client applications.